PRACTICAL 1

- Create different blinking patterns with LEDs by connecting to GPIO pints of Raspberry Pi.

Aim: To display Different LED pattern with raspberry Pi.

Hardware Requirements:

- Raspberry pi

- LEDs

- Resistors

- Jumper wire (Male-Female)

Code:

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BOARD)

GPIO.setwarnings(False)

GPIO.setup(7,GPIO.OUT)

GPIO.setup(29,GPIO.OUT)

GPIO.setup(31,GPIO.OUT)

GPIO.setup(33,GPIO.OUT)

while(1):

GPIO.output(7,False)

print("LED 1 IS OFF")

time.sleep(1)

GPIO.output(29,False)

print("LED 2 IS OFF")

time.sleep(1.5)

GPIO.output(31,False)

print("LED 3 IS OFF")

time.sleep(2)

GPIO.output(33,False)

print("LED 4 IS OFF")

time.sleep(2.5)

GPIO.output(7,True)

print("LED 1 IS FINALLY ON")

time.sleep(3.5)

GPIO.output(29,True)

print("LED 2 IS FINALLY ON")

time.sleep(4)

GPIO.output(31,True)

print("LED 3 IS FINALLY ON")

time.sleep(4.5)

GPIO.output(33,True)

print("LED 4 IS FINALLY ON")

time.sleep(5)

GPIO.output(7,False)

print("LED 1 IS OFF")

time.sleep(1)

GPIO.output(29,False)

print("LED 2 IS OFF")

time.sleep(1.5)

GPIO.output(31,False)

print("LED 3 IS OFF")

time.sleep(2)

GPIO.output(33,False)

print("LED 4 IS OFF")

time.sleep(2.5)

print("PROGRAM COMPLETE!")

GPIO.cleanup()

Output:

PRACTICAL 2

- Use Telegram to control Raspberry Pi GPIO

Aim: Controlling Raspberry pi using Telegram.

Hardware Requirements:

- Raspberry Pi

- LEDs

- Register

- Breadboard

- Jumper Wire (Male-Female)

- Mobile Phone

Steps to perform on Telegram:

- Install Telegram App On your Smart phone.

- Open Telegram Search for botfather and click on start or type /start.

- Provide a name for your bot

E.g: “tyitIOT”

- Then provide username for your bot. Note that bot must end with _bot.

- After this process the botfather give a token number for accessing, copy pate those token number in the code.

Steps to perform on raspberry command Prompt:

- Sudo apt-update

- Sudo apt-get install python-pip

- Sudo pip install teleport

- Go to python idle3 & open telegram.py (or the folder where you save the code)

Code:

import time, datetime

import RPi.GPIO as GPIO

import telepot

from telepot.loop import MessageLoop

first_led = 7

second_led = 29

third_led = 31

fourth_led = 33

now = datetime.datetime.now()

GPIO.setmode(GPIO.BOARD)

GPIO.setwarnings(False)

#LED first_led

GPIO.setup(first_led, GPIO.OUT)

GPIO.output(first_led, 0) #Off initially

#LED second_led

GPIO.setup(second_led, GPIO.OUT)

GPIO.output(second_led, 0) #Off initially

#LED third_led

GPIO.setup(third_led, GPIO.OUT)

GPIO.output(third_led, 0) #Off initially

##LED fourth_led

GPIO.setup(fourth_led, GPIO.OUT)

GPIO.output(fourth_led, 0) #Off initially

def action(msg):

chat_id = msg['chat']['id']

command = msg['text']

print ('Received: %s' % command)

if 'on' in command:

message = "on"

if 'first_led' in command:

message = message + "first_led"

GPIO.output(first_led, 1)

if 'second_led' in command:

message = message + "second_led"

GPIO.output(second_led, 1)

if 'third_led' in command:

message = message + "third_led"

GPIO.output(third_led, 1)

if 'fourth_led' in command:

message = message + "fourth_led"

GPIO.output(fourth_led, 1)

if 'all' in command:

GPIO.output(first_led, 1)

GPIO.output(second_led, 1)

GPIO.output(third_led, 1)

GPIO.output(fourth_led, 1)

message = message + "light(s)"

telegram_bot.sendMessage (chat_id, message)

if 'off' in command:

message = "off "

if 'first_led' in command:

message = message + "first_led"

GPIO.output(first_led, 0)

if 'second_led' in command:

message = message + "second_led"

GPIO.output(second_led, 0)

if 'third_led' in command:

message = message + "third_led"

GPIO.output(third_led, 0)

if 'fourth_led' in command:

message = message + "fourth_led"

GPIO.output(fourth_led, 0)

if 'all' in command:

message = message + "all"

GPIO.output(first_led, 0)

GPIO.output(second_led, 0)

GPIO.output(third_led, 0)

GPIO.output(fourth_led, 0)

message = message + "light(s)"

telegram_bot.sendMessage (chat_id, message)

telegram_bot = telepot.Bot('1993353550:AAHT4l_R0YXCsanE0MtOGLtMPy4mQEMMtY0')

print (telegram_bot.getMe())

MessageLoop(telegram_bot, action).run_as_thread()

print ('Up and Running....')

while 1:

time.sleep(10)

Output:

PRACTICAL 3

- Interface Pi Camera with Raspberry Pi to capture image and video.

Aim: Capturing image with Raspberry Pi and Pi camera.

Hardware Requirements:

- Raspberry pi

- Pi Camera

Note:

1. Start the Raspberry Pi.

2. Open the Raspberry Pi Configuration tool from the main menu.

3. Enable the camera software if it is disabled.

4. Reboot the Raspberry Pi to apply the changes.

5. The camera can also be enabled using the command:

"sudo raspi-config"

Commands to download pi camera libraries:

- Sudo apt-get install python3_pip

- Pip3 install picamera

- Then go to python environment & then open picam5 (or the folder where you save the code).

Code:

from picamera2 import Picamera2

import time

picam2 = Picamera2()

picam2.configure(picam2.create_video_configuration())

output_path = "/home/admin/Desktop/ty7.jpg"

try:

picam2.start_recording(output_path)

print("Recording started...")

time.sleep(5)  # record 5 seconds

finally:

picam2.stop_recording()

picam2.close()  # release camera safely

print("Camera released, recording saved:", output_path)

Output:

PRACTICAL 4

- Write a Python program to display time on a 4-digit 7-segment display using Raspberry Pi.

Aim: To display time over a 4-digit 7-segment display using Raspberry Pi.

Hardware Requirements:

- Raspberry Pi

- TM1637-4-digit 7-segment display board

- Female to female jumping wires

Connections:

| Raspberry Pi | 4-digit 7-segment |
| --- | --- |
| Pin2(5V) | Vcc pin |
| Pin6 (GND) | GND pin |
| Pin38 | DIO |
| Pin40 | CLK |

Note:

- To download the libraries open web browser on your Raspberry Pi and log on to the following link:

http://github.com/timwaizenegger/raspberrypi-example/tree/master/actor-led-7-segment-4numbers

- Click on the actor-led-7segment-4numbers zip folders and now click on download button to download the file.

- Now on your RPi move to /home/pi/Download.

- Unzip the file and try to execute different example codes present in the folder.

- Open python2 IDLE-create a new file-write the code given and save it in the same folder i.e. actor-led-7segment-4numbers since the code depends on tm1637.py

Code:

#!/usr/bin/env python

# -*- coding: utf-8 -*-

from time import sleep

impor tm1637

try:

import thread

except ImportError:

import _thread as thread

#Initialization the clock (GND, VCC=3.3V, Example pins are DIO-20 and CLK21)

Display = tm1637.TM1637(CLK=21, DIO=20, brightness=1.0)

try:

print("Starting clock in the background press CTRL + C to stop:")

Display.StartClock(military_time=False)

Print("Continue Python Script and tweak Display!")

sleep(5)

Display.ShowDoublepoint(False)

sleep(5)

loops = 3

while loops > 0:

for i in range(0, 10):

Display.setBrightness(i/ 10.0)

sleep(0.5)

loops -= 1

Display.StopClock()

thread.interrupt_main()

except keyboardInterrupt:

print("Properly closing the clock and open GPIO pins")

Display.cleanup()

Output: